LaneSight

MASTER PRODUCT + ENGINEERING CONTEXT

Canonical build document for coding, AI handoff, product decisions and MVP execution

| Principle | Canonical decision |
| --- | --- |
| What LaneSight is | A hardware-agnostic bowling intelligence platform that turns POV video, plus optional sensor data, into shot telemetry and coaching. |
| Initial input | Meta smart glasses may be the first convenient capture device, but Meta is only an input adapter. Any suitable POV video must work. |
| MVP | Single intentionally captured POV shot video → lane calibration → ball tracking → metrics → pin result → intended-vs-actual feedback. |
| MVP+ | Wearable capture, sensor fusion, multi-shot sessions, lane-transition detection, coaching, league/local-event modes. |
| Primary markets | Practice/self-analysis, coaching, league bowling where permitted, and potentially organizer-approved local tournaments. |
| Competition stance | Do not make the business dependent on PBA/elite competition use. Build rules-aware modes and post-session analysis. |
| Privacy stance | No continuous ambient recording by default. Intentional shot capture, data minimization, no facial recognition, source-video deletion. |
| Core technical gate | Can POV video reconstruct the ball path accurately enough that serious bowlers trust it? |

Version 2.0 | September 2026 | Treat this document as source-of-truth until superseded

# 0. How to Use This Document

This document is intentionally written as both a product specification and an AI/developer context package. Give it to ChatGPT, Claude, Codex or an engineer before implementation. If a later conversation conflicts with this document, explicitly record the new decision and update this source-of-truth rather than silently changing architecture.

## 0.1 Copy/paste AI kickoff prompt

You are working on LaneSight. Treat the attached LaneSight Master Product + Engineering Context as the canonical product specification. Do not redesign the product assumptions unless asked. Prioritize the MVP technical proof: a single intentional POV bowling-shot video must be calibrated, tracked, converted into physical lane coordinates, measured, confidence-scored, and returned as useful output. Keep all capture hardware behind adapters and all measurements source-aware so future sensors can override or fuse with vision estimates. Before coding, identify the specific milestone, files/components affected, acceptance criteria, and tests. Never fabricate a bowling metric when confidence is insufficient.

# 1. Product Vision

LaneSight is the intelligence layer for bowling shots. Cameras provide visual evidence, optional sensors provide high-fidelity release telemetry, and LaneSight combines those inputs into an understandable reconstruction of what happened and, as the product matures, what the bowler should do next.

## 1.1 Product promise

Works from a valid POV bowling video regardless of camera brand.

Measures where the ball traveled, not merely the final score.

Uses optional sensors to improve accuracy rather than making special hardware mandatory.

Explains confidence and refuses to manufacture precision.

Separates measured facts from inferred lane behavior and coaching recommendations.

Supports improvement, practice and coaching first; competition functionality is rules-aware.

## 1.2 Product language

| Layer | Question answered |
| --- | --- |
| Vision | Where did the ball go? |
| Sensor | How was the ball released / rotating? |
| Context | What was the bowler trying to do and what equipment/pattern was used? |
| Intelligence | Why did the result happen and how does it compare with prior shots? |
| Coaching | What should the bowler change next, when enough evidence exists? |

# 2. Product Scope & Users

| Use case | Capabilities | Position |
| --- | --- | --- |
| Practice / self-analysis | Full metrics, history, comparisons, coaching, sensor fusion | Primary |
| Coaching | Student sessions, objective telemetry, comparisons, reports | Primary / B2B |
| League | Live analysis/features only where permitted; configurable mode | Primary expansion |
| Local tournament | Organizer/rules-approved functionality; restricted modes possible | Expansion |
| Elite/PBA competition | Do not assume live coaching is allowed; practice/warm-up/post-session value remains | Not required for business viability |

Do not market LaneSight as a way to circumvent competition rules. Build functionality so capture, metrics, recommendations, audio and live coaching can be independently enabled/disabled by mode.

# 3. Product Architecture Principles

Hardware agnostic: the analysis engine consumes normalized video, not 'Meta video'.

Sensor optional: vision-only must remain useful.

Source-aware metrics: every measurement stores source, confidence and version.

Sensor fusion ready: authoritative sensor measurements can supplement or supersede weaker visual estimates while retaining provenance.

Deterministic measurements before generative coaching.

Offline/reproducible metric engine: formulas should be testable without UI or cloud.

Privacy by design: intentionally captured shots, minimum useful frames, cropped lane ROI, no facial recognition.

Adapter boundaries: camera, sensor, storage and coaching providers are replaceable.

Model/version traceability: every result is reproducible from input + code/model versions.

# 4. MVP Definition

The MVP proves the analysis engine, not the glasses experience. A user supplies one intentionally captured POV video containing one bowling shot. LaneSight converts that video into an annotated reconstruction and useful measurements.

| MVP capability | Required? |
| --- | --- |
| Upload/import POV video | YES |
| Validate video/FPS/resolution | YES |
| Automatic lane calibration | YES, with manual fallback |
| Ball detection + temporal tracking | YES |
| Pixel → board/downlane mapping | YES |
| Trajectory smoothing | YES |
| Launch board / launch angle | YES |
| Speed | YES |
| Arrow board + intended-target error | YES |
| Breakpoint board + distance | YES |
| Entry board + angle | YES |
| Pin result | YES; strike/non-strike minimum |
| Per-metric confidence / unavailable state | YES |
| Top-down path + annotated replay | YES |
| Save result + delete raw source independently | YES |
| RPM from vision | Experimental; visible-marker path acceptable |
| Smart-glasses integration | NO - MVP+ |
| Lane transition / live move recommendation | NO - requires multi-shot context |
| Opponent scouting | NO - later |

## 4.1 MVP definition of done

Supported held-out clips complete without developer repair at least 90% of the time.

Median trajectory lateral error is approximately ≤1 board under supported conditions.

Speed is within ±0.5 mph against a trusted reference under supported conditions.

Arrow and entry board median error ≤1 board; breakpoint target ~1–1.5 boards when visible.

Every metric has confidence and provenance; low-quality metrics become unavailable.

User sees annotated replay, top-down path, metrics, result and intended-vs-actual observations.

Raw source video can be deleted without deleting the derived shot record.

# 5. MVP+ Roadmap

| Feature | Phase |
| --- | --- |
| Meta / wearable direct capture adapter | MVP+ |
| Tap/gesture to arm; automatic release detection; auto-stop at impact | MVP+ |
| Audio post-shot feedback | MVP+ |
| Other smart-glasses / action-camera adapters | MVP+ |
| F5/Storm or other bowling-sensor integration | MVP+ |
| Multi-shot sessions + consistency | MVP+ |
| Lane-transition detection from comparable shots | MVP+ |
| Feet/target/speed adjustments | MVP+ |
| Ball-change recommendations | MVP+ |
| Arsenal + oil-pattern intelligence | MVP+ |
| Coach dashboard / multi-athlete | MVP+ |
| League mode / rules profile | MVP+ |
| Competition-restricted/post-session mode | MVP+ |
| Warm-up scouting / anonymous other-bowler trajectories | Later, explicit mode only |
| Axis tilt/rotation from vision | Later unless independently validated |

# 6. Input Contract

NormalizedVideoInput {
  input_id
  source_type: UPLOAD | META | PHONE | GOPRO | OTHER_POV
  video_uri
  fps
  width
  height
  duration_ms
  timestamps_available
  camera_intrinsics?      # if supplied
  distortion_parameters? # if supplied
  device_metadata?
}

ShotContext {
  handedness: RIGHT | LEFT
  intended_arrow_board?
  intended_breakpoint_board?
  intended_breakpoint_distance_ft?
  ball_id?
  oil_pattern_id?
  lane_number?
  session_id?
}

The analysis service must not contain Meta-specific assumptions. Device-specific capture/metadata conversion occurs before NormalizedVideoInput.

# 7. Physical Coordinate System

Use a camera-independent lane plane. Board numbering follows standard physical convention: board 1 at the right edge viewed from foul line toward pins; board 39 at the left. Handedness changes interpretation language, not stored coordinates.

lane_width_in = 41.5
lane_width_ft = 41.5 / 12 = 3.45833
board_count = 39
board_width_in = 41.5 / 39 ≈ 1.0641
board_width_ft ≈ 0.088675
foul_line_y = 0 ft
arrow_reference_y ≈ 15 ft
headpin_reference_y = 60 ft

TrajectoryPoint {
  frame
  timestamp_ms
  pixel_x
  pixel_y
  board_x
  distance_ft
  detection_confidence
}

# 8. Computer-Vision Pipeline

INPUT VIDEO
  → normalize codec / timestamps
  → select lane region
  → lens correction when calibration metadata exists
  → lane landmark detection
  → homography / perspective mapping
  → ball detection
  → temporal ball tracking
  → map detections into lane coordinates
  → trajectory smoothing / interpolation
  → release / arrow / breakpoint / entry / impact events
  → metric engine
  → pin-state analysis
  → confidence gates
  → sensor fusion (when available)
  → result JSON + annotated video + top-down lane plot
  → single-shot observation engine

## 8.1 Calibration

Estimate a planar homography from image landmarks to known lane-plane coordinates. Use at least four non-collinear correspondences; prefer additional landmarks and robust fitting. Automatic detection should use lane edges/gutters, foul-line or near-lane references, pin deck/headpin and optionally arrows/dots. Manual calibration is a required fallback.

[u v 1]^T ~ H [x y 1]^T   → normalize homogeneous scale → physical lane coordinate

## 8.2 Ball detection/tracking

Fine-tune an object detector on POV bowling frames; prioritize tiny downlane-ball performance.

Track detections temporally with motion constraints and a Kalman/ByteTrack-style approach.

Reject impossible motion and retain raw detections separately from smoothed path.

Bridge short occlusions with reduced confidence; long gaps invalidate affected metrics.

Create a developer overlay showing detection boxes/centers, mapped points and confidence.

## 8.3 Trajectory

Fit a smooth x=f(y) trajectory using a robust smoothing spline or comparable method. Avoid high-order polynomial oscillation. Calculations use the fitted trajectory while raw points remain auditable.

# 9. Metric Engine

| Metric | Definition / calculation | Preferred source |
| --- | --- | --- |
| Launch board | Interpolate x(y) at standardized early point, initially y=1 ft | Vision |
| Launch angle | Early-path tangent; atan(Δx_ft/Δy_ft) | Vision |
| Arrow board | Interpolate x at y=15 ft | Vision |
| Target error | actual_arrow_board - intended_arrow_board | Vision + user intent |
| Speed | Physical path distance / elapsed time; mph = ft/s × 0.681818 | Sensor if trusted; else vision |
| Breakpoint | Credible lateral extremum where fitted dx/dy changes sign | Vision |
| Breakpoint distance | Downlane y at breakpoint | Vision |
| Entry board | Interpolate x at standardized 60-ft reference | Vision |
| Entry angle | Final path tangent over robust ~4–8 ft window | Vision |
| Speed loss | Early segment speed - entry segment speed | Sensor/vision depending availability |
| RPM | rotations/time ×60 or 60×FPS/frames-per-rotation | Sensor preferred; vision experimental |
| Axis tilt / rotation | Release/rotation telemetry | Sensor preferred; vision later |
| Pin result | Standing-pin state after impact | Vision |

## 9.1 Core formulas

Δx_ft = (board2 - board1) × 0.088675
Δy_ft = distance2 - distance1
distance_ft = sqrt(Δx_ft² + Δy_ft²)
speed_ft_s = distance_ft / Δt_seconds
speed_mph = speed_ft_s × 0.681818

Interpolation at y*:
α = (y* - y1)/(y2 - y1)
x* = x1 + α(x2 - x1)

Entry tangent:
slope_ft_per_ft = f'(y) × 0.088675
entry_angle_deg = atan(abs(slope_ft_per_ft)) × 180/π

RPM:
rpm = rotations / elapsed_seconds × 60
or rpm = 60 × FPS / frames_per_rotation

Estimate speeds and tangents from multiple observations/regression windows rather than adjacent frames; adjacent-frame pixel noise can create large errors.

# 10. Measurement Provenance & Sensor Fusion

This is a hard architectural requirement. Never store a naked metric value without knowing how it was produced.

Measurement {
  metric_type
  value
  unit
  source: VISION | SENSOR_F5 | SENSOR_OTHER | USER | FUSED
  confidence: 0.0..1.0
  source_timestamp?
  algorithm_version?
  sensor_device_id?
  raw_reference?
}

## 10.1 Fusion rules

Do not average measurements blindly.

Define source-specific accuracy profiles by metric.

A trusted sensor may be authoritative for RPM/axis metrics while vision remains authoritative for lane path.

When both measure speed, retain both raw values, calculate disagreement, and expose the selected/fused value plus provenance.

Use sensor measurements as development ground truth only after independently validating sensor accuracy/behavior.

Design integrations around a generic SensorAdapter interface so Storm/F5 is a flagship opportunity, not a hard dependency or exclusive architecture.

## 10.2 Sensor adapter contract

SensorAdapter {
  connect()
  identifyDevice()
  startSession()
  getShotEvents()
  normalizeMeasurement()
  syncShot(videoShotTimestamp)
}

SensorShot {
  timestamp
  speed?
  rpm?
  axis_rotation?
  axis_tilt?
  flare?
  loft?
  vendor_raw_payload?
}

# 11. Single-Shot Output & Coaching Boundaries

The MVP can provide execution feedback from one shot, but should not claim lane transition or prescribe a lane move from an isolated shot without sufficient baseline evidence.

| Evidence | Allowed output |
| --- | --- |
| Actual vs intended arrow | “Crossed 1.2 boards right of target.” |
| Actual vs intended breakpoint | “Breakpoint was 1.4 boards inside and 2.0 ft early.” |
| Poor measurement quality | “Breakpoint could not be measured reliably.” |
| One isolated shot | Describe execution/result; do not claim oil moved. |
| Comparable multi-shot history | MVP+ may infer earlier/later reaction and recommend adjustments with evidence. |

# 12. App Experience

## 12.1 MVP screens

Home / Analyze Shot.

Video import/capture.

Shot context: handedness; intended target; optional ball/pattern.

Calibration review/manual correction when required.

Processing stages: calibrating → tracking → calculating → pins → complete.

Result: annotated replay + top-down trajectory.

Metric cards with confidence.

Intended vs actual line.

Observation card.

Shot history/detail.

Privacy controls including Delete Source Video.

## 12.2 MVP+ wearable UX

READY → explicit ARM → RECORD → RELEASE DETECTED → BALL TRACKING → PIN IMPACT → STOP → ANALYZE → READY

Normal bowling sessions should not use continuous full-resolution ambient recording. The user explicitly arms capture immediately before the shot via supported gesture/button, voice or app/watch fallback. The system automatically stops after impact. Smart glasses should use their normal visible recording indicator; LaneSight must not circumvent it.

## 12.3 Operating modes

| Mode | Purpose |
| --- | --- |
| Training | Full telemetry, sensor fusion, history and coaching. |
| League | Capabilities independently configurable based on applicable rules/league policy. |
| Competition Restricted | Disable live recommendations/audio/other disallowed assistance; retain permitted capture or post-session analysis where allowed. |
| Scouting | Separate explicit future mode; never silently repurpose normal personal-shot capture to record other bowlers. |

# 13. Privacy, Battery & Other-Bowler Protection

Intent-triggered recording is the default product behavior for wearables.

Record only the shortest window necessary for the user's shot; stop after pin impact.

Crop/mask adjacent lanes, settee/spectator areas and faces where feasible in derived artifacts.

No facial recognition, identity inference or biometric templates.

Do not identify other bowlers in normal mode.

Analyze-and-delete should be available: retain derived trajectory/metrics while deleting raw video.

Do not use user footage for training without an explicit consent/product policy.

Heavy CV should run on phone/cloud where practical rather than continuously on glasses, reducing wearable battery/heat.

Future scouting requires explicit activation and rules/venue/privacy consideration.

# 14. Seed Dataset & Training Strategy

Existing personal bowling videos are immediately useful as LaneSight Seed Dataset v1, but should first validate geometry/calculations rather than train a model that overfits one bowler, center, ball set or camera.

datasets/
  raw/
    session_YYYYMMDD/
      shot_0001.mp4
      shot_0002.mp4
  annotations/
    lane_landmarks/
    ball_centers/
    pin_states/
    shot_metadata/
  splits/
    train.json
    validation.json
    test.json

## 14.1 Per-shot annotation

ShotAnnotation {
  shot_id
  bowler_id
  handedness
  camera/source
  center/session
  ball
  oil_pattern?
  intended_target?
  known_speed?
  known_rpm?
  result
  lane_landmarks[]
  ball_points[{frame,x,y}]
  pin_state?
  notes?
}

## 14.2 Dataset expansion

Phase 1: 10–20 personal POV clips; manually calibrate and click ball centers to prove math.

Phase 2: 50–100 personal shots across balls/lines; automate tracking and quantify error.

Phase 3: add 10+ bowlers, left/right handedness, multiple centers, balls, lighting and camera devices.

Phase 4: collect synchronized sensor/reference data to create trusted labels.

Split evaluation by session/center/bowler, not random adjacent frames, to prevent leakage.

# 15. Recommended Tech Stack

| Layer | Recommended starting choice |
| --- | --- |
| Mobile | React Native + Expo; move device-specific capture into native modules/adapters when required |
| Developer/debug web | Next.js / React |
| Analysis API | Python FastAPI |
| CV | OpenCV + PyTorch |
| Detection | Benchmark YOLO-family / RT-DETR-style detectors; fine-tune on LaneSight data |
| Tracking | Kalman + assignment / ByteTrack-style tracker |
| Numerics | NumPy + SciPy |
| Video | FFmpeg + OpenCV |
| DB/Auth | Supabase Postgres + Auth |
| Object storage | Supabase Storage initially; S3-compatible abstraction |
| Async jobs | Simple background worker locally; Redis/Celery or managed queue as load grows |
| Deployment | Local Apple Silicon/GPU during R&D; GPU worker only when needed |
| Observability | Structured logs + Sentry + model/version metadata |
| Source control | GitHub monorepo |

# 16. Repository Structure

lanesight/
├── README.md
├── docs/
│   ├── PRODUCT_CONTEXT.md
│   ├── ARCHITECTURE.md
│   ├── METRICS.md
│   └── DATASET.md
├── apps/
│   ├── mobile/
│   └── debug-web/
├── services/
│   └── analysis-api/
│       ├── api/
│       ├── video/
│       ├── calibration/
│       ├── detection/
│       ├── tracking/
│       ├── trajectory/
│       ├── metrics/
│       ├── pins/
│       ├── fusion/
│       ├── observations/
│       └── tests/
├── packages/
│   ├── contracts/
│   ├── camera-adapters/
│   └── sensor-adapters/
├── datasets/
│   ├── schemas/
│   └── tools/
├── models/
│   └── configs/
└── infra/

# 17. Core Data Model

| Entity | Purpose |
| --- | --- |
| User | Account/profile |
| BowlerProfile | Handedness and bowling preferences |
| Ball | Arsenal/equipment |
| OilPattern | Pattern metadata |
| Session | Practice/league/coaching context |
| Shot | One bowling delivery |
| VideoAsset | Source/derived clip metadata and retention |
| Calibration | Homography/landmark result |
| TrajectoryPoint | Raw/mapped/smoothed path data |
| Measurement | Value + unit + source + confidence + version |
| PinState | Result / standing pins |
| SensorDevice | Optional connected measurement device |
| SensorShot | Vendor-normalized release telemetry |
| Observation | Deterministic intended-vs-actual finding |
| Recommendation | MVP+ evidence-backed coaching action |
| TransitionState | MVP+ multi-shot lane behavior |

# 18. API Surface

| Endpoint | Purpose |
| --- | --- |
| POST /analysis | Create analysis job |
| POST /analysis/{id}/video | Attach/complete normalized video input |
| POST /analysis/{id}/context | Shot intent/equipment context |
| POST /analysis/{id}/calibration | Manual calibration fallback |
| GET /analysis/{id} | Status + final result |
| POST /analysis/{id}/correction | Correct pins/calibration/approved labels |
| DELETE /analysis/{id}/source-video | Delete raw video independently |
| POST /sensor-events | MVP+ normalized sensor-shot ingestion |
| POST /sessions | MVP+ multi-shot session |
| GET /sessions/{id}/transition | MVP+ lane-transition state |

# 19. Result Contract

AnalysisResult {
  shot_id
  status
  input: {source_type, fps, width, height, duration_ms}
  calibration: {quality, reprojection_error, version}
  trajectory: {
    raw_points[],
    smoothed_points[],
    quality
  }
  measurements: Measurement[]
  pin_result: {strike, standing_pins[], confidence}
  observations: [{
    type,
    text,
    evidence,
    confidence
  }]
  artifacts: {
    annotated_video_uri,
    lane_plot_uri,
    thumbnail_uri
  }
  versions: {
    pipeline,
    detector,
    tracker,
    metric_engine
  }
}

# 20. Engineering Validation

| Capability | Initial target |
| --- | --- |
| Trajectory | Median lateral error ≤1 board; also report p90 |
| Speed | ±0.5 mph vs trusted reference |
| Arrow board | Median error ≤1 board |
| Breakpoint | ~1–1.5 boards and ~2 ft when clear |
| Entry board | Median error ≤1 board |
| Entry angle | ~1° target after validation |
| RPM vision | ±30 RPM only in supported visible-feature conditions; otherwise unavailable |
| Strike/non-strike | ≥95% on held-out supported clips |
| Latency | <10 sec post-upload target on MVP compute, secondary to accuracy |

## 20.1 Ground-truth strategy

The strongest development environment is synchronized POV video plus independent references. A future sensor partner can provide release telemetry; a trusted lane-tracking/reference system can validate path. Never assume either is ground truth without checking its measurement behavior. Store reference and LaneSight outputs side by side.

# 21. Implementation Plan - Start Here

| Milestone | Build | Exit criterion |
| --- | --- | --- |
| M0 - Repo + contracts | Monorepo, Python service, shared schemas, sample video loader | One command processes a local video job skeleton |
| M1 - Manual geometry | Manual lane landmarks + homography + top-down warp | Known lane points map correctly |
| M2 - Manual trajectory | Developer clicks ball center through one clip | All metric formulas work from human-labeled path |
| M3 - Metric engine | Launch/speed/arrows/breakpoint/entry + unit tests | Synthetic + manual trajectories produce expected metrics |
| M4 - Auto detection | Ball detector + tracker + debug overlay | Automatic trajectory approaches manual ground truth |
| M5 - Confidence | Quality gates + unavailable states | Bad clips fail safely |
| M6 - Pins | Strike/non-strike + correction | Held-out target reached |
| M7 - API/storage | Upload → async analysis → result JSON/artifacts | End-to-end backend works |
| M8 - MVP UI | Import/context/process/results/history/privacy | Tester can use without developer |
| M9 - Validation | 50–100+ held-out shots + error report | Core accuracy targets demonstrated |
| M10 - Sensor/wearable adapters | Only after core engine proof | Inputs can change without analysis-engine rewrite |

# 22. First Coding Backlog

Create GitHub monorepo and README containing product non-negotiables.

Define Pydantic/JSON schemas for NormalizedVideoInput, ShotContext, TrajectoryPoint, Measurement and AnalysisResult.

Create Python package for lane constants and coordinate conversions.

Create a CLI: `analyze-shot input.mp4 --annotation shot.json`.

Build frame extractor preserving timestamps/FPS.

Build a simple calibration UI/script where the developer clicks lane landmarks.

Implement homography and render a top-down lane image.

Build manual ball-center annotation tool for one shot.

Implement metric functions as pure/testable Python.

Create synthetic trajectory unit tests before detector work.

Generate annotated video and top-down trajectory artifact.

Label the first 10–20 personal clips.

Only after manual pipeline works: benchmark detector candidates and train/fine-tune ball detection.

# 23. Non-Negotiable Tests

Straight synthetic path must not invent a breakpoint.

Known constant-speed path returns correct speed.

Known 15-ft crossing returns correct arrow board.

Known final tangent returns correct entry angle after board-to-feet conversion.

Short tracking gap lowers confidence; long gap invalidates affected metric.

Implausible homography is rejected.

No visible rotation evidence returns RPM unavailable, not guessed.

All sensor-vs-vision conflicts preserve both raw measurements and selected provenance.

Deleting source video does not corrupt derived shot data.

# 24. Product Decisions That Must Stay Explicit

| Decision | Current answer |
| --- | --- |
| Is Meta required? | No. It is an initial capture adapter only. |
| Is a sensor required? | No. Optional and accuracy-enhancing. |
| Preferred sensor partnership? | Storm/F5 is strategically attractive; architecture remains vendor-agnostic. |
| Can personal videos start development? | Yes. Use immediately for seed/validation; avoid overfitting. |
| Continuous recording? | No by default. Intent-triggered shot capture. |
| Record other bowlers? | Not in normal mode. Future explicit scouting mode only. |
| Live elite competition use? | Do not depend on it. Build rules-aware/restricted modes. |
| Can one shot prove lane transition? | No. |
| Should an LLM calculate metrics? | No. CV/geometry/math calculate; generative layer may explain evidence later. |
| Should low-confidence metrics be shown? | Only clearly confidence-gated; unavailable is preferable to false precision. |

# 25. Commercial Direction (Context for Product Decisions)

LaneSight should be positioned as bowling improvement and coaching technology, not primarily as an in-competition advantage. The initial consumer funnel can work without smart glasses: upload a POV video and receive analysis. Wearables reduce capture friction; sensors improve telemetry; coaching and multi-shot intelligence deepen recurring value.

Consumer: practice/self-analysis subscription.

Premium: wearable/live training experience and multi-shot intelligence.

Coach/pro-shop: multi-bowler dashboard and reporting.

Potential partners: sensor manufacturers, ball manufacturers, training centers and coaching programs.

Do not create hardware lock-in or advertising-driven recommendations that compromise coaching trust.

# 26. What Not to Build Before the Core Proof

Complex smart-glasses UX before POV analysis accuracy is demonstrated.

Continuous session surveillance.

Opponent identity or facial recognition.

Lane-oil maps presented as directly measured oil.

AI-generated lane moves without comparable-shot evidence.

Axis tilt/rotation from vision without validation.

Large-scale social/community features.

Premature subscription/paywall complexity.

Vendor-exclusive architecture.

# 27. Immediate Next Action

Start with one real POV video. Manually identify lane landmarks and manually mark the ball center across frames. Produce the top-down trajectory and calculate every MVP metric from that manually verified path. This isolates the geometry/math problem from the object-detection problem and creates the first ground-truth record. Once those outputs are correct, automate ball detection and tracking.

The first engineering milestone is therefore not 'connect Meta glasses.' It is: **given one valid POV MP4, reproduce the ball's physical path accurately and deterministically.**

# Appendix A - AI Handoff Checklist

Read this document before proposing architecture.

State which milestone you are implementing.

Preserve hardware/sensor adapter boundaries.

Do not change coordinate conventions silently.

Keep metric calculations pure, deterministic and unit-tested.

Store raw and smoothed trajectory separately.

Attach provenance/confidence to every measurement.

Do not infer lane transition from one shot.

Do not guess RPM/axis metrics when visual evidence is insufficient.

Update documentation/contracts whenever a canonical decision changes.

# Appendix B - Future Sensor Partnership Design Goal

A flagship sensor integration should create a 'complete shot' rather than duplicate LaneSight. Sensor: release/rotation telemetry. LaneSight Vision: lane path and outcome. LaneSight Intelligence: relationship between release, path, result and subsequent adjustments. This separation keeps the platform useful with or without the partner and gives the partner incremental value rather than positioning LaneSight as a replacement.